# ATTENDRO: Smart Biometric + App-Based Attendance Management System using AI & IoT

A Project Report Submitted by

[STUDENT NAME]

In partial fulfillment of the requirements for the Diploma in

APPLIED AI & ML

At

Rajarambapu Institute of Technology, Islampur

2025–2026

Under the Guidance of

[GUIDE NAME]

# CERTIFICATE

This is to certify that the project titled “ATTENDRO: Smart Biometric + App-Based Attendance Management System using AI & IoT” has been carried out by [Student Name] under my guidance and supervision in partial fulfillment of the requirements for the award of the Diploma in Applied AI & ML at Rajarambapu Institute of Technology, Islampur, during the academic year 2025–2026.

| ___________________ Guide | ___________________ H.O.D. | ___________________ Principal |
| --- | --- | --- |

# ACKNOWLEDGEMENT

I express sincere gratitude to **[Guide Name]** for guidance and feedback, to the **Department of Applied AI & ML** for providing laboratory resources, to my group members for their dedicated collaboration, and to my parents for their unwavering support throughout this project.

# TABLE OF CONTENTS

Title Page ............................................................... i

Certificate of the Guide ................................................ ii

Acknowledgement ......................................................... iii

Index / Contents ........................................................ iv

Abstract ............................................................... v

List of Figures ........................................................ vi

List of Tables ......................................................... vii

Chapter–1 Introduction ................................................. 1

Chapter–2 Literature Survey ............................................. 5

Chapter–3 Scope of the Project .......................................... 9

Chapter–4 Methodology / Approach ........................................ 13

Chapter–5 Designs, Working and Processes ............................... 18

Chapter–6 Results and Applications ...................................... 27

Chapter–7 Conclusion ................................................... 32

References ............................................................. 34

# ABSTRACT

This project presents **Attendro**, an intelligent, portable, and offline-first biometric attendance system designed for educational institutes. Addressing the limitations of fixed biometric terminals and proxy-prone manual registers, Attendro introduces a **session-controlled** architecture where attendance can only be marked during active, authorized lecture sessions. The system integrates an **ESP32-based portable device** with a fingerprint sensor, a **Supabase (Cloud)** backend for real-time synchronization, and a **Faculty Web App** for session management.

Key innovations include **Time-Variant Batch Locking** (ensuring students only mark attendance for their specific batch), **Offline-First Synchronization** (queueing scans when Wi-Fi is unavailable), and **Context-Aware AI Rules** that validate scans against subject, class, and schedule constraints locally. This system satisfies AIML diploma requirements by leveraging biometric pattern recognition and rule-based decision intelligence to ensure data integrity and operational efficiency.

# LIST OF FIGURES

1. System Architecture Diagram – Figure 1

2. Database Schema (ER Diagram) – Figure 2

3. User Workflow & Experience – Figure 3

4. Device Interface & Wiring – Figure 4

5. Security Model & Session Token Flow – Figure 5

6. Use Case Diagram – Figure 6

7. Context-Level DFD – Figure 7

# LIST OF TABLES

1. Pin Configuration (ESP32 to R307/OLED) – Table 1

2. Security Threat Matrix – Table 2

3. Database Entities Description – Table 3

---

# CHAPTER–1 INTRODUCTION

## 1.1 PROBLEM STATEMENT

Polytechnic institutes and colleges today face significant challenges in attendance management:

1.  **Proxy Attendance:** Manual rolls are easily manipulated, and QR-code systems can be shared remotely.

2.  **Lack of Context:** Standard biometric machines allow "punching in" at any time, even if the student skips the actual lecture.

3.  **Data Fragmentation:** Attendance data is often siloed in physical registers, delaying the generation of defaulter lists and compliance reports.

4.  **Fixed Infrastructure:** Wall-mounted biometric devices are expensive to install in every classroom and laboratory.

## 1.2 OBJECTIVES

The primary objective is to design and build a **Portable, Session-Controlled Biometric Attendance System** that ensures attendance is taken only during an active lecture session for the correct subject, class, and batch.

Specific sub-objectives include:

Portable Hardware:** Develop an ESP32-based battery-powered device that moves with the faculty.

Session Control:** Implement "Session Tokens" to lock attendance to specific time windows.

Offline Resilience:** Ensure the system works without active Internet, syncing data when connectivity returns.

AI Integration:** Incorporate biometric pattern recognition and rule-based AI for context verification.

Real-Time Analytics:** Provide instant access to attendance stats via a web dashboard.

## 1.3 CONSTRAINTS AND ASSUMPTIONS

The device relies on periodic Wi-Fi connectivity for synchronization.

Fingerprint templates are stored securely and matched either on-device or on-server depending on mode.

Faculty are responsible for charging the portable devices and initiating sessions via their smartphones.

---

# CHAPTER–2 LITERATURE SURVEY

Biometric Systems:** Traditional fixed systems provide high accuracy but lack schedule awareness (Ross & Jain, 2021). Students can mark attendance and leave class.

IoT Attendance:** Recent IEEE papers discuss IoT-enabled attendance, but many lack offline queuing mechanisms, leading to data loss in unstable networks.

Session-Based Tokenization:** Secure systems in banking use time-limited tokens; we adapt this for attendance, generating a unique token for every lecture slot.

Gap Analysis:** Existing solutions are either purely software (easy to spoof) or purely hardware (dumb terminals). Attendro bridges this by making the hardware "context-aware" via a cloud connection.

---

# CHAPTER–3 SCOPE OF THE PROJECT

## 3.1 OVERVIEW

The project scope covers the end-to-end development of hardware, firmware, cloud infrastructure, and a web application.

## 3.2 IN-SCOPE DELIVERABLES

1.  **Portable Device Firmware (ESP32):**

Wi-Fi connectivity and Session Polling.

Fingerprint acquisition and Template generation.

Local Rule Engine (Context checks).

Offline Queue Management and Synchronization logic.

2.  **Central Cloud (Supabase):**

SQL Schema for Users, Classes, and Attendance.

Edge Functions for `start_session`, `mark_attendance`, and `sync_offline`.

Row-Level Security (RLS) to protect flexible data.

3.  **Faculty/Admin Web Interface:**

Dashboard for Timetable and Analytics.

Session Control (Start/End).

Reporting Module (Defaulters, PDF Export).

## 3.3 OUT-OF-SCOPE

Advanced face recognition or iris scanning.

Payroll processing integration.

Hardware enclosure manufacturing (prototype uses acrylic/3D print case).

---

# CHAPTER–4 METHODOLOGY / APPROACH

## 4.1 SYSTEM OVERVIEW

Figure 1: System Architecture

┌─────────────────────────────────────────────────────────────────────────────────────────────────────┐
│                                      ATTENDRO SYSTEM ARCHITECTURE                                    │
│                    Teacher-Unlocked, Subject-Wise Biometric Attendance System                       │
└─────────────────────────────────────────────────────────────────────────────────────────────────────┘

┌──────────────────────────────────────────────────────────────────────────────────────────────────────┐
│                                         FACULTY LAYER                                                 │
├──────────────────────────────────────────────────────────────────────────────────────────────────────┤
│                                                                                                       │
│    ┌─────────────────────────────────────┐      ┌─────────────────────────────────────┐              │
│    │       FACULTY MOBILE APP            │      │       FACULTY WEB APP               │              │
│    │    ┌───────────────────────────┐    │      │    ┌───────────────────────────┐    │              │
│    │    │  📱 Start Lecture Session │    │      │    │  💻 Session Management    │    │              │
│    │    │  📋 Select Subject        │    │      │    │  📊 Live Attendance View  │    │              │
│    │    │  👥 Select Class/Batch    │    │      │    │  📈 Reports & Analytics   │    │              │
│    │    │  ⏰ Set Duration          │    │      │    │  ⚙️  Settings             │    │              │
│    │    │  🔐 Generate Session Token│    │      │    │  📝 Session History       │    │              │
│    │    └───────────────────────────┘    │      │    └───────────────────────────┘    │              │
│    └─────────────────────────────────────┘      └─────────────────────────────────────┘              │
│                         │                                        │                                    │
│                         └────────────────┬───────────────────────┘                                    │
│                                          ▼                                                            │
└──────────────────────────────────────────┼───────────────────────────────────────────────────────────┘
                                           │ HTTPS/REST API
┌──────────────────────────────────────────┼───────────────────────────────────────────────────────────┐
│                                   CLOUD BACKEND LAYER                                                │
├──────────────────────────────────────────┼───────────────────────────────────────────────────────────┤
│                                          ▼                                                           │
│    ┌─────────────────────────────────────────────────────────────────────────────────────────────┐   │
│    │                              SUPABASE BACKEND                                                │   │
│    │  ┌────────────────────────────────────────────────────────────────────────────────────────┐ │   │
│    │  │                            EDGE FUNCTIONS (APIs)                                        │ │   │
│    │  │  ┌──────────────────┐  ┌──────────────────┐  ┌──────────────────┐  ┌────────────────┐  │ │   │
│    │  │  │  create-session  │  │  verify-token    │  │  sync-attendance │  │  get-students  │  │ │   │
│    │  │  │  ───────────────  │  │  ─────────────   │  │  ───────────────  │  │  ─────────────  │  │ │   │
│    │  │  │  • Generate Token │  │  • Validate JWT  │  │  • Batch Insert  │  │  • By Class    │  │ │   │
│    │  │  │  • Set Batch Lock │  │  • Check Expiry  │  │  • Conflict Res  │  │  • By Batch    │  │ │   │
│    │  │  │  • Time Window    │  │  • Device Auth   │  │  • Offline Merge │  │  • Roll List   │  │ │   │
│    │  │  └──────────────────┘  └──────────────────┘  └──────────────────┘  └────────────────┘  │ │   │
│    │  └────────────────────────────────────────────────────────────────────────────────────────┘ │   │
│    │                                          │                                                   │   │
│    │  ┌────────────────────────────────────────────────────────────────────────────────────────┐ │   │
│    │  │                              POSTGRESQL DATABASE                                        │ │   │
│    │  │                                                                                         │ │   │
│    │  │  ┌─────────────────┐  ┌─────────────────┐  ┌──────────────────┐  ┌──────────────────┐  │ │   │
│    │  │  │    students     │  │    subjects     │  │ biometric_sessions│  │biometric_records │  │ │   │
│    │  │  │  ─────────────  │  │  ─────────────  │  │  ───────────────  │  │ ───────────────   │  │ │   │
│    │  │  │  • roll_no      │  │  • subject_code │  │  • session_token │  │  • roll_no       │  │ │   │
│    │  │  │  • fingerprint_id│  │  • name        │  │  • device_id     │  │  • session_id    │  │ │   │
│    │  │  │  • class_id     │  │  • type (TH/PR) │  │  • allowed_rolls │  │  • timestamp     │  │ │   │
│    │  │  │  • batch        │  │  • semester     │  │  • batch_lock    │  │  • sync_status   │  │ │   │
│    │  │  └─────────────────┘  └─────────────────┘  └──────────────────┘  └──────────────────┘  │ │   │
│    │  │                                                                                         │ │   │
│    │  │  ┌─────────────────┐  ┌─────────────────┐  ┌──────────────────┐  ┌──────────────────┐  │ │   │
│    │  │  │    classes      │  │    faculty      │  │   esp32_devices  │  │  offline_queue   │  │ │   │
│    │  │  │  ─────────────  │  │  ─────────────  │  │  ───────────────  │  │ ───────────────   │  │ │   │
│    │  │  │  • division     │  │  • profile_id   │  │  • device_id     │  │  • record_data   │  │ │   │
│    │  │  │  • year         │  │  • department   │  │  • location      │  │  • created_at    │  │ │   │
│    │  │  │  • semester     │  │  • emp_code     │  │  • status        │  │  • synced_at     │  │ │   │
│    │  │  └─────────────────┘  └─────────────────┘  └──────────────────┘  └──────────────────┘  │ │   │
│    │  └────────────────────────────────────────────────────────────────────────────────────────┘ │   │
│    │                                                                                              │   │
│    │  ┌────────────────────────────────────────────────────────────────────────────────────────┐ │   │
│    │  │                              REALTIME SUBSCRIPTIONS                                     │ │   │
│    │  │        WebSocket channels for live attendance updates to dashboard                      │ │   │
│    │  └────────────────────────────────────────────────────────────────────────────────────────┘ │   │
│    └─────────────────────────────────────────────────────────────────────────────────────────────┘   │
│                                          │                                                           │
└──────────────────────────────────────────┼───────────────────────────────────────────────────────────┘
                                           │ HTTPS/REST + WebSocket
                                           ▼
┌──────────────────────────────────────────────────────────────────────────────────────────────────────┐
│                                      IOT DEVICE LAYER                                                │
├──────────────────────────────────────────────────────────────────────────────────────────────────────┤
│                                                                                                       │
│    ┌─────────────────────────────────────────────────────────────────────────────────────────────┐   │
│    │                              ESP32 BIOMETRIC DEVICE                                          │   │
│    │                                                                                              │   │
│    │  ┌─────────────────────────────────────────────────────────────────────────────────────┐    │   │
│    │  │                              HARDWARE COMPONENTS                                      │    │   │
│    │  │                                                                                       │    │   │
│    │  │   ┌─────────────────────┐    ┌─────────────────────┐    ┌─────────────────────┐     │    │   │
│    │  │   │   ESP32 DevKit V1   │    │  R307 Fingerprint   │    │  0.93" OLED SPI     │     │    │   │
│    │  │   │   ─────────────────  │    │  Sensor             │    │  Display            │     │    │   │
│    │  │   │   • Wi-Fi Module    │    │  ─────────────────   │    │  ─────────────────   │     │    │   │
│    │  │   │   • Dual Core CPU   │◄──►│  • 120 ID Storage   │    │  • 128x64 pixels    │     │    │   │
│    │  │   │   • 520KB SRAM      │    │  • UART Interface   │    │  • SPI Interface    │     │    │   │
│    │  │   │   • Flash Storage   │    │  • FAR < 0.001%     │    │  • Status Display   │     │    │   │
│    │  │   │   • GPIO Pins       │────►│  • FRR < 0.1%       │    │  • User Feedback    │     │    │   │
│    │  │   └─────────────────────┘    └─────────────────────┘    └─────────────────────┘     │    │   │
│    │  │              │                                                    ▲                  │    │   │
│    │  │              └────────────────────────────────────────────────────┘                  │    │   │
│    │  │                              SPI Communication                                        │    │   │
│    │  └─────────────────────────────────────────────────────────────────────────────────────┘    │   │
│    │                                                                                              │   │
│    │  ┌─────────────────────────────────────────────────────────────────────────────────────┐    │   │
│    │  │                              SOFTWARE MODULES                                         │    │   │
│    │  │                                                                                       │    │   │
│    │  │   ┌─────────────────┐   ┌─────────────────┐   ┌─────────────────┐   ┌─────────────┐  │    │   │
│    │  │   │ Session Manager │   │ Fingerprint     │   │ Offline Queue   │   │ Display     │  │    │   │
│    │  │   │ ─────────────── │   │ Handler         │   │ Manager         │   │ Controller  │  │    │   │
│    │  │   │ • Token Verify  │   │ ─────────────── │   │ ─────────────── │   │ ─────────── │  │    │   │
│    │  │   │ • Roll List     │   │ • Scan & Match  │   │ • Store Local   │   │ • Show State│  │    │   │
│    │  │   │ • Time Check    │   │ • ID Extraction │   │ • Auto Sync     │   │ • Show Roll │  │    │   │
│    │  │   │ • Batch Filter  │   │ • Validation    │   │ • Conflict Res  │   │ • Errors    │  │    │   │
│    │  │   └─────────────────┘   └─────────────────┘   └─────────────────┘   └─────────────┘  │    │   │
│    │  └─────────────────────────────────────────────────────────────────────────────────────┘    │   │
│    └─────────────────────────────────────────────────────────────────────────────────────────────┘   │
│                                                                                                       │
└──────────────────────────────────────────────────────────────────────────────────────────────────────┘

┌──────────────────────────────────────────────────────────────────────────────────────────────────────┐
│                                      DATA FLOW LEGEND                                                │
├──────────────────────────────────────────────────────────────────────────────────────────────────────┤
│                                                                                                       │
│    ────────►  HTTP/HTTPS Request           ════════►  WebSocket/Realtime                            │
│    ◄────────  HTTP/HTTPS Response          ◄════════  Subscription Updates                          │
│    ─ ─ ─ ─►  Offline Queue Sync            ▬▬▬▬▬▬▬►  Hardware Interface                             │
│                                                                                                       │
│    🔐 Session Token Flow:  Faculty App → Supabase → ESP32 Device                                    │
│    👆 Fingerprint Flow:    R307 Sensor → ESP32 → Supabase (or Local Queue)                          │
│    📊 Realtime Updates:    Supabase → Dashboard (WebSocket)                                         │
│                                                                                                       │
└──────────────────────────────────────────────────────────────────────────────────────────────────────┘

The system comprises three main interconnected components:

1.  **Portable Device:** Handheld unit carried by faculty to class. It serves as the capture point.

Hardware:* ESP32 Controller, R307 Fingerprint Sensor, 0.96" OLED Display, Rechargeable Battery.

Identity:* Each device has a burned-in, unique `device_code`.

2.  **Central Cloud (Supabase):** The brain of the system.

Stores all Master Data (Students, Subjects) and Transaction Data (Sessions, Logs).

Runs "Edge Functions" (Serverless API) to handle complex logic securely.

3.  **Faculty App (Web):** The control center.

Allows faculty to log in, select their current class/subject, and "Start Session" on a specific device code.

## 4.2 DEVICE LOGIC & WORKFLOW

The device operates in a continuous state machine loop:

1.  **Boot:** Connects to Wi-Fi and displays its `device_code`.

2.  **Poll:** Periodically checks the Cloud for an "Active Session" linked to its ID.

3.  **Session Active:** Once a session starts, it locks to that Class/Subject. The OLED displays "Marketing: [Subject]".

4.  **Scan & Capture:**

Student places finger.

Device captures image -> extracts Template.

Device verifies **Context Rules** locally (Is this batch allowed? Duplicate scan?).

5.  **Sync:**

Online:** Pushes data immediately to Cloud Edge Function.

Offline:** Saves scan to onboard memory (Queue) with timestamp. Pushes when Wi-Fi returns.

## 4.3 OFFLINE-FIRST & SYNC STRATEGY

To ensure robustness in campus environments with spotty Wi-Fi:

Local Queue:** The ESP32 maintains a circular buffer of attendance records.

Deferred Matching:** If the server cannot be reached, the scan is stored. Upon reconnection, the queue is flushed in batches.

Integrity Check:** Records are only deleted from the device queue after receiving a positive `200 OK` acknowledgement from the server. This guarantees zero data loss.

## 4.4 AI & AIML JUSTIFICATION

This project meets AIML diploma requirements through:

1.  **Biometric Pattern Recognition:** The core sensor performs feature extraction (minutiae points) and template matching, a fundamental pattern recognition task.

2.  **Context-Aware Rule-Based Intelligence:** The device implements a "Classical AI" rule engine (If-Then logic) to make autonomous decisions:

If* Current Time > End Time → Reject.

If* Roll No is not in Batch A → Reject.

If* Student already marked → Reject.

3.  **Behavioral Analysis (Future/Server-side):** collected timestamp data can be analyzed to detect anomalies, such as "Rapid Fire Scans" indicating a potential bypass attempt.

---

# CHAPTER–5 DESIGNS, WORKING AND PROCESSES

## 5.1 HARDWARE DESIGN

Controller:** ESP32 DevKit V1 (Dual-core, Wi-Fi/BLE).

Biometric Sensor:** R307 / Waveshare Optical Sensor (UART interface).

Display:** SSD1306 I2C OLED (Visual feedback for students).

Power:** Li-Ion Battery with TP4056 charging module.

## 5.2 DATABASE DESCRIPTION (SUPABASE)

Figure 2: Database Schema

┌─────────────────────────────────────────────────────────────────────────────────────────────────────┐
│                                 ATTENDRO DATABASE SCHEMA (ER DIAGRAM)                                │
│                                   PostgreSQL / Supabase Database                                     │
└─────────────────────────────────────────────────────────────────────────────────────────────────────┘

                                    ┌────────────────────────────┐
                                    │        PROFILES            │
                                    │────────────────────────────│
                                    │ PK id: UUID                │
                                    │    name: TEXT              │
                                    │    role: USER_ROLE         │
                                    │    department: TEXT        │
                                    │    created_at: TIMESTAMPTZ │
                                    │    updated_at: TIMESTAMPTZ │
                                    └─────────────┬──────────────┘
                                                  │
                                                  │ 1:1
                                                  ▼
┌──────────────────────────────┐    ┌────────────────────────────┐
│         CLASSES              │    │         FACULTY            │
│──────────────────────────────│    │────────────────────────────│
│ PK id: UUID                  │    │ PK id: UUID                │
│    name: TEXT                │    │ FK profile_id: UUID ───────┼────────────────────┐
│    year: INTEGER             │    │    employee_code: TEXT     │                    │
│    semester: INTEGER         │    │    designation: TEXT       │                    │
│    division: TEXT            │    │    department: TEXT        │                    │
│    department: TEXT          │    │    status: TEXT            │                    │
│ FK class_teacher_id: UUID ───┼───►│    created_at: TIMESTAMPTZ │                    │
│    created_at: TIMESTAMPTZ   │    │    updated_at: TIMESTAMPTZ │                    │
│    updated_at: TIMESTAMPTZ   │    └────────────────────────────┘                    │
└───────────────┬──────────────┘                  │                                    │
                │                                  │                                    │
                │ 1:N                              │ 1:N                                │
                ▼                                  ▼                                    │
┌──────────────────────────────┐    ┌────────────────────────────────────────────────┐ │
│         STUDENTS             │    │            SUBJECT_ALLOCATIONS                  │ │
│──────────────────────────────│    │────────────────────────────────────────────────│ │
│ PK id: UUID                  │    │ PK id: UUID                                    │ │
│    name: TEXT                │    │ FK class_id: UUID ─────────────────────────────┼─┼──┐
│    enrollment_no: TEXT       │    │ FK subject_id: UUID ───────────────────────────┼─┼──┼──┐
│    roll_no: INTEGER ◄────────┼────│ FK faculty_id: UUID ───────────────────────────┼─┘  │  │
│    year: INTEGER             │    │    academic_year: TEXT                         │    │  │
│    semester: INTEGER         │    │    semester: INTEGER                           │    │  │
│ FK class_id: UUID ───────────┼───►│    batch: TEXT (A/B/ALL) ◄─── NEW              │    │  │
│    division: TEXT            │    │    created_at: TIMESTAMPTZ                     │    │  │
│    batch: TEXT (A/B) ◄───────┼────│────────────────────────────────────────────────│    │  │
│    department: TEXT          │    └────────────────────────────────────────────────┘    │  │
│    fingerprint_id: INT ◄─────┼──── NEW: Maps to R307 sensor ID (1-120)                  │  │
│    mobile: TEXT              │                                                          │  │
│    email: TEXT               │    ┌────────────────────────────────────────────────┐    │  │
│    status: STUDENT_STATUS    │    │             SUBJECTS                            │◄───┼──┘
│    created_at: TIMESTAMPTZ   │    │────────────────────────────────────────────────│    │
│    updated_at: TIMESTAMPTZ   │    │ PK id: UUID                                    │    │
└───────────────┬──────────────┘    │    subject_code: TEXT                          │    │
                │                    │    name: TEXT                                  │    │
                │                    │    semester: INTEGER                           │    │
                │                    │    year: INTEGER                               │    │
                │                    │    department: TEXT                            │    │
                │                    │    type: SUBJECT_TYPE (TH/PR/TU)               │    │
                │                    │    weekly_lectures: INTEGER                    │    │
                │                    │    status: TEXT                                │    │
                │                    │    created_at: TIMESTAMPTZ                     │    │
                │                    │    updated_at: TIMESTAMPTZ                     │    │
                │                    └────────────────────────────────────────────────┘    │
                │                                                                          │
                │                    ┌────────────────────────────────────────────────┐    │
                │                    │           ESP32_DEVICES (NEW)                  │    │
                │                    │────────────────────────────────────────────────│    │
                │                    │ PK id: UUID                                    │    │
                │                    │    device_id: TEXT (unique hardware ID)        │    │
                │                    │    location: TEXT (classroom/lab)              │    │
                │                    │    status: TEXT (ACTIVE/INACTIVE)              │    │
                │                    │    last_ping: TIMESTAMPTZ                      │    │
                │                    │    firmware_version: TEXT                      │    │
                │                    │    created_at: TIMESTAMPTZ                     │    │
                │                    └─────────────────┬──────────────────────────────┘    │
                │                                      │                                    │
                │                                      │ 1:N                                │
                │                                      ▼                                    │
                │                    ┌────────────────────────────────────────────────┐    │
                │                    │        BIOMETRIC_SESSIONS (NEW)                │◄───┘
                │                    │────────────────────────────────────────────────│
                │                    │ PK id: UUID                                    │
                │                    │    session_token: TEXT (JWT/unique key)        │
                │                    │ FK device_id: UUID ────────────────────────────┼────┐
                │                    │ FK faculty_id: UUID ───────────────────────────┼────┼─►
                │                    │ FK class_id: UUID ─────────────────────────────┼────┤
                │                    │ FK subject_id: UUID ───────────────────────────┼────┤
                │                    │    lecture_type: TEXT (THEORY/PRACTICAL)       │    │
                │                    │    batch_lock: TEXT (A/B/ALL)                  │    │
                │                    │    allowed_rolls: INTEGER[] (roll list)        │    │
                │                    │    start_time: TIMESTAMPTZ                     │    │
                │                    │    end_time: TIMESTAMPTZ                       │    │
                │                    │    duration_minutes: INTEGER                   │    │
                │                    │    status: TEXT (ACTIVE/COMPLETED/CANCELLED)   │    │
                │                    │    created_at: TIMESTAMPTZ                     │    │
                │                    └─────────────────┬──────────────────────────────┘    │
                │                                      │                                    │
                │                                      │ 1:N                                │
                │                                      ▼                                    │
                │                    ┌────────────────────────────────────────────────┐    │
                │                    │        BIOMETRIC_RECORDS (NEW)                 │    │
                │                    │────────────────────────────────────────────────│    │
                └───────────────────►│ PK id: UUID                                    │    │
                                     │ FK session_id: UUID ───────────────────────────┼────┤
                                     │ FK student_id: UUID                            │    │
                                     │    roll_no: INTEGER (from fingerprint_id)      │    │
                                     │    fingerprint_id: INTEGER (matched ID)        │    │
                                     │    status: ATTENDANCE_STATUS                   │    │
                                     │    scan_timestamp: TIMESTAMPTZ                 │    │
                                     │    sync_status: TEXT (SYNCED/PENDING/FAILED)   │    │
                                     │    device_id: TEXT                             │    │
                                     │    verification_score: INTEGER (0-100)         │    │
                                     │    remarks: TEXT                               │    │
                                     │    created_at: TIMESTAMPTZ                     │    │
                                     │    synced_at: TIMESTAMPTZ                      │    │
                                     └────────────────────────────────────────────────┘    │
                                                                                           │
                                     ┌────────────────────────────────────────────────┐    │
                                     │        OFFLINE_SYNC_QUEUE (NEW)                │    │
                                     │────────────────────────────────────────────────│    │
                                     │ PK id: UUID                                    │    │
                                     │ FK device_id: UUID ────────────────────────────┼────┘
                                     │    session_token: TEXT                         │
                                     │    record_data: JSONB                          │
                                     │    attempts: INTEGER                           │
                                     │    last_attempt: TIMESTAMPTZ                   │
                                     │    status: TEXT (PENDING/PROCESSING/DONE/FAIL) │
                                     │    error_message: TEXT                         │
                                     │    created_at: TIMESTAMPTZ                     │
                                     │    processed_at: TIMESTAMPTZ                   │
                                     └────────────────────────────────────────────────┘


┌─────────────────────────────────────────────────────────────────────────────────────────────────────┐
│                                         ENUM DEFINITIONS                                             │
├─────────────────────────────────────────────────────────────────────────────────────────────────────┤
│                                                                                                      │
│   USER_ROLE          STUDENT_STATUS      SUBJECT_TYPE      ATTENDANCE_STATUS      SYNC_STATUS       │
│   ──────────         ──────────────      ────────────      ─────────────────      ───────────       │
│   • ADMIN            • ACTIVE            • TH (Theory)     • PRESENT              • SYNCED          │
│   • FACULTY          • YD (Year Down)    • PR (Practical)  • ABSENT               • PENDING         │
│   • STUDENT          • PASSOUT           • TU (Tutorial)   • LATE                 • FAILED          │
│                                                             • REJECTED                               │
│                                                                                                      │
│   SESSION_STATUS          BATCH_TYPE           LECTURE_TYPE                                         │
│   ──────────────          ──────────           ────────────                                         │
│   • ACTIVE                • A                  • THEORY                                              │
│   • COMPLETED             • B                  • PRACTICAL                                           │
│   • CANCELLED             • ALL                • TUTORIAL                                            │
│   • EXPIRED                                                                                          │
│                                                                                                      │
└─────────────────────────────────────────────────────────────────────────────────────────────────────┘


┌─────────────────────────────────────────────────────────────────────────────────────────────────────┐
│                                     RELATIONSHIP SUMMARY                                             │
├─────────────────────────────────────────────────────────────────────────────────────────────────────┤
│                                                                                                      │
│  PROFILES ──────────────────────── 1:1 ────────────────────────► FACULTY                            │
│  CLASSES ───────────────────────── 1:N ────────────────────────► STUDENTS                           │
│  CLASSES ───────────────────────── 1:N ────────────────────────► SUBJECT_ALLOCATIONS                │
│  SUBJECTS ──────────────────────── 1:N ────────────────────────► SUBJECT_ALLOCATIONS                │
│  FACULTY ───────────────────────── 1:N ────────────────────────► SUBJECT_ALLOCATIONS                │
│  FACULTY ───────────────────────── 1:N ────────────────────────► BIOMETRIC_SESSIONS                 │
│  ESP32_DEVICES ─────────────────── 1:N ────────────────────────► BIOMETRIC_SESSIONS                 │
│  BIOMETRIC_SESSIONS ────────────── 1:N ────────────────────────► BIOMETRIC_RECORDS                  │
│  STUDENTS ──────────────────────── 1:N ────────────────────────► BIOMETRIC_RECORDS                  │
│  ESP32_DEVICES ─────────────────── 1:N ────────────────────────► OFFLINE_SYNC_QUEUE                 │
│                                                                                                      │
└─────────────────────────────────────────────────────────────────────────────────────────────────────┘


┌─────────────────────────────────────────────────────────────────────────────────────────────────────┐
│                              FINGERPRINT ID MAPPING STRATEGY                                         │
├─────────────────────────────────────────────────────────────────────────────────────────────────────┤
│                                                                                                      │
│   RULE: fingerprint_id = roll_no (Direct 1:1 Mapping)                                               │
│                                                                                                      │
│   ┌─────────────────────────────────────────────────────────────────────────────────────────────┐   │
│   │   STUDENTS TABLE                           R307 SENSOR STORAGE                              │   │
│   │   ────────────────                         ──────────────────                               │   │
│   │                                                                                              │   │
│   │   Roll 1  ─────── fingerprint_id: 1  ─────► Sensor ID #1 (Template)                         │   │
│   │   Roll 2  ─────── fingerprint_id: 2  ─────► Sensor ID #2 (Template)                         │   │
│   │   Roll 37 ─────── fingerprint_id: 37 ─────► Sensor ID #37 (Template)                        │   │
│   │   Roll 120 ────── fingerprint_id: 120 ────► Sensor ID #120 (Template)                       │   │
│   │                                                                                              │   │
│   │   Benefits:                                                                                  │   │
│   │   ✓ Simple lookup on ESP32 (no complex mapping table)                                       │   │
│   │   ✓ Direct roll number identification from scan result                                      │   │
│   │   ✓ Easy reporting and debugging                                                            │   │
│   │   ✓ Scalable for 120 students per class                                                     │   │
│   └─────────────────────────────────────────────────────────────────────────────────────────────┘   │
│                                                                                                      │
└─────────────────────────────────────────────────────────────────────────────────────────────────────┘

The database is normalized to ensure consistency:

`devices`: Stores `device_code`, `status`, and `last_seen_at`.

`lecture_sessions`: The core entity connecting `faculty_id`, `device_code`, `subject_id`, `class_id`, `batch_id`, and `session_token`.

`students`: Contains `roll_no`, `fingerprint_template` (optional), and mapping to multiple classes.

`attendance_records`: Use `session_id` and `roll_no` as unique composite keys to prevent duplicates.

`finger_templates`: Central repository for "Enroll Once, Use Anywhere".

## 5.3 SESSION & CONTEXT RULES (THE VERIFICATION LOGIC)

A scan is only accepted if it passes the following **Context Rules**:

1.  **Time Window:** `Current Time` must be between `session.start` and `session.end`.

2.  **Device Match:** The provided `session_token` matches the device's current active session.

3.  **Batch Lock:** If the session is for "Batch A" (e.g., Roll 1-20), a student with Roll 25 is rejected.

4.  **De-Duplication:** Checks if `attendance_records` already contains this `roll_no` for this `session_id`.

## 5.4 USE CASE SCENARIOS

**Scenario A: Normal Lecture**

1. Faculty enters class, opens Web App.

2. Selects "Third Year", "Data Structures", "Theory".

3. Clicks "Start Session".

4. Device beeps and shows "Data Structures Active".

5. Device is passed around; students scan.

6. Faculty clicks "End Session". Report is generated instantly.

**Scenario B: No Internet**

1. Session starts (while online or cached).

2. Internet cuts out.

3. Students continue scanning; device shows "Saved Offline".

4. Faculty returns to staff room (Wi-Fi restored).

5. Device automatically flushes queue; server processes records.

---

# CHAPTER–6 RESULTS AND APPLICATIONS

## 6.1 INNOVATION & NOVELTY

1.  **Session-Controlled Attendance:** Unlike standard biometrics that accept any finger at any time, Attendro only unlocks for specific, authorized windows.

2.  **Portable & Wire-Free:** The battery-powered design allows it to be used in labs, seminar halls, or playgrounds without wiring.

3.  **Smart Batch Locking:** Enforces attendance rules strictly (e.g., stopping Batch B students from attending Batch A labs).

4.  **Hybrid AI Approach:** Combines edge-based rule intelligence with cloud-based analytics.

## 6.2 APPLICATION AREAS

Polytechnic & Engineering Colleges:** For theory lectures, practical labs, and workshops.

Seminar Halls:** For tracking attendance at guest lectures.

Examination Halls:** For verifying student identity before exams.

## 6.3 EXPECTED RESULTS

Accuracy:** >98% True Acceptance Rate on fingerprints.

Speed:** <2 seconds per student scan-to-verify.

Efficiency:** Reduces attendance taking time from 10 minutes (manual) to ~2 minutes (parallel scanning).

---

# CHAPTER–7 CONCLUSION

Attendro successfully demonstrates how IoT and AI can modernize a legacy process. By decoupling the biometric sensor from the wall and connecting it to a session-aware cloud brain, we eliminate proxy attendance and automate the tedious task of compliance reporting. The system is offline-tolerant, secure, and user-friendly, making it a viable product for educational institutions.

---

# REFERENCES

[1] Ross, A., & Jain, A. (2021). "Biometric sensor interoperability". *IEEE Trans. on PAMI*.

[2] Espressif Systems. (2023). "ESP32 Technical Reference Manual".

[3] Supabase. (2024). "Realtime & Edge Functions Documentation".

[4] "R307 Optical Fingerprint Sensor Datasheet", Hangzhou Grow Technology.

---

*Diagrams referenced in List of Figures are available in the project documentation folder.*